FIFO Verification

Using SystemVerilog

Mahmoud Ghamry

SEP 2024

Table of Contents

Introduction                                                                                                                                                 1

FIFO Architecture                                                                                                                                        2

Verification Flow                                                                                                                                         4

Verification Plan                                                                                                                                          4

Design           											                    5

Bugs                                            									                    6

Verification Files                                                                                                                                          7

Interface                                                                                                                          7

Share_pkg                                                                                                                       7

Top                                                                                                     		         8

Transaction                                           						         9

Testbench                                                                                                                      10

Monitor                      									      11

Coverage                  									      12

Scoreboard 										      13

Design After Edit Bugs and Assertion						      16

Do file and Assertion                                                                                                  18

Result                                                                                                                                                          19

Trascript                                                                                                                                          19

Function Coverage											      19

Assertion												      19

Cross Coverage                                         							      20

Waveform                                                                                                                   		      20

1.Introduction

First-In-First-Out (FIFO) buffers are fundamental components in digital systems, serving as temporary storage areas that manage the flow of data between producer and consumer processes. Their inherent design allows data to be stored in the order it is received, ensuring that the first data to enter the buffer is the first to be retrieved. This characteristic makes FIFOs essential in various applications, including data streaming, buffering between asynchronous systems, and handling variable data rates.

As the complexity of digital systems increases, ensuring the correctness and reliability of FIFO implementations becomes paramount. Verification is a critical phase in the design process, aimed at identifying and resolving potential issues before deployment. A rigorous verification strategy not only validates the functional correctness of the FIFO but also checks for edge cases, performance metrics, and adherence to specifications.

This document explores the methodologies employed in verifying FIFO designs using SystemVerilog, a powerful hardware description and verification language. By leveraging SystemVerilog's capabilities, we can create comprehensive testbenches, cover a wide range of scenarios, and ensure the FIFO operates reliably under all conditions. Through systematic testing and validation, we aim to guarantee that the FIFO meets its design goals and performs efficiently in real-world applications.

1

2.FIFO Architecture

The architecture of a FIFO buffer typically consists of several key components that work together to facilitate data storage, retrieval, and management. Below is a description of the primary elements that make up a FIFO design:

1. Data Storage Array

- Description: The core component of the FIFO is a memory array that holds the data elements. This array is usually implemented using registers or RAM blocks.

- Functionality: The size of the array determines the capacity of the FIFO. The data is written to the next available location in the array and read from the oldest location.

2. Write Pointer

- Description: A pointer that indicates the next available position in the FIFO for writing new data.

- Functionality: The write pointer increments each time data is written, wrapping around to the beginning of the array when it reaches the end (circular buffer behavior).

3. Read Pointer

- Description: A pointer that indicates the location of the next data element to be read from the FIFO.

- Functionality: Similar to the write pointer, the read pointer increments each time data is read, wrapping around when reaching the end of the array.

4. Control Logic

- Description: A set of logic circuits that manage the behavior of the FIFO, including writing, reading, overflow, and underflow conditions.

- Functionality:

  - Write Enable (wr_en): Signals whether a write operation can occur.

  - Read Enable (rd_en): Signals whether a read operation can occur.

2

  - Full and Empty Flags: Indicators that signal whether the FIFO is full or empty, respectively. These flags are essential for preventing overflow and underflow conditions.

5. Status Indicators

- Description: Additional signals that provide information about the FIFO's state.

- Functionality:

  - Full Flag: Set when the write pointer reaches the read pointer after wrapping around.

  - Empty Flag: Set when the write pointer equals the read pointer, indicating no data is available to read.

  - Almost Full / Almost Empty Flags: These may be included to provide early warning conditions for high-level control mechanisms.

6. Data Input/Output Interfaces

- Description: Interfaces for connecting the FIFO to other components in the system.

- Functionality: This includes input lines for writing data into the FIFO and output lines for reading data out of the FIFO.

3

3. Verification Flow

4. Verification Plan

4

5.Design

5

6.Bugs

- The underflow flag should be registered.

- Reset the flags ( empty –  almostempty –  wr_ack – full –  alomstfull ).

- Add cases

    - If the wr_en , rd_en and empty is high so the write is the higher priority  and counter decreases .

    - If the wr_en , rd_en and full is high so the Read is the higher priority and counter increases .

    - If the wr_en and rd_en is high and not full nor empty so the two cases run together but the counter not changed.

    - If the wr_en and rd_en is low so everything not changed.

- The almostfull must be high if the FIFO need one input to be high

So I edit it

6

7. Verification Files

7.1 interface

7.2 Share_pkg

7

7.3 Top

8

7.4 Transaction

9

7.5 Testbench

10

7.6 Monitor

11

7.7 Coverage

12

7.8 Scoreboard

13

14

15

7.9 Design After Edit Bugs and Assertion

16

17

Assertion

Do file

18

8.Result

8.1Trascript

8.2 Function Coverage

8.3 Assertion

19

8.4 Cross Coverage

8.5 Waveform

20

Go to my Github to show more : repo

21